Steel and Wood Shapes

## Type Catalogue

- Steel shapes are generated from type catalogues.

- Type catalogues are text files with ANSI encoding.

- The text file is a comma delimitated, with no spaces.  Commas are used to separate fields.

- A return is needed at the end of each row.

Description

The first row of the text file maps to parameters in Revit families.  For example, the below parameter “d” maps to the depth of the I-Shape beam.

Syntax:

- Header field syntax:

Where

| d | Revit Parameter |
| --- | --- |
| ## | Character Separator |
| length | Dimension |
| millimeters | Unit |
| , | Field Separator |

- The first column must contain a comma.  This comma indicates that the type name (steel member size) is located in that column.

- Successive rows of data must be sequential with the header row.

I-Shape (W Shape)

Use M_W-Wide Flange.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,tw##length##millimeters,bf##length##millimeters,tf##length##millimeters,k##length##millimeters

| Syntax Header | Description | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters, | Thickness of Flange | millimeters |
| k##length##millimeters, | Distance from outer flange edge to end or radius | millimeters |

M-Shape (Miscellaneous Wide Flange)

Use M_M-Miscellaneous Wide Flange.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,tw##length##millimeters,bf##length##millimeters,k##length##millimeters

| Syntax Header | Description | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters, | Thickness of Flange | millimeters |
| k##length##millimeters | Distance from outer flange edge to end or radius | millimeters |

HP-Shape (HP-Bearing Pile)

Use M_HP-Bearing Pile.txt as template

Header

,W##other##,A##area##millimeters,d##length##millimeters,tw##length##millimeters,bf##length##millimeters,tf##length##millimeters, k##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters | Thickness of Flange | millimeters |
| k##length##millimeters | Distance from outer flange edge to end or radius | millimeters |

C-Shape (Channel)

Use M_HP-Bearing Pile.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,tw##length##millimeters,bf##length##millimeters,tf##length##millimeters,x##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters, | Thickness of Flange | millimeters |
| x##length##millimeters | Distance from edge of web to centroid | millimeters |

MC-Shape (Miscellaneous Channel)

Use M_MC-Miscellaneous Channel.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,tw##length##millimeters,bf##length##millimeters,tf##length##millimeters,x##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters, | Thickness of Flange | millimeters |
| x##length##millimeters | Distance from edge of web to centroid | millimeters |

ST- Structural Tee

Use M_ST-Structural Tee.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,bf##length##millimeters,tw##length##millimeters,tf##length##millimeters,k##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters, | Thickness of Flange | millimeters |
| k##length##millimeters | Distance from outer flange edge to end or radius | millimeters |

MT- Structural Tee

Use M_MT-Structural Tee.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,bf##length##millimeters,tw##length##millimeters,tf##length##millimeters,k##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters, | Thickness of Flange | millimeters |
| k##length##millimeters | Distance from outer flange edge to end or radius | millimeters |

WT- Structural Tee

Use M_WT-Structural Tee.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,bf##length##millimeters,tw##length##millimeters,tf##length##millimeters,k##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| tw##length##millimeters, | Thickness of Web | millimeters |
| bf##length##millimeters, | Width of Flange | millimeters |
| tf##length##millimeters, | Thickness of Flange | millimeters |
| k##length##millimeters | Distance from outer flange edge to end or radius | millimeters |

L-Angle

Use M_L-Angle.txt as template

,W##other##,A##area##millimeters,d##length##millimeters,b##length##millimeters,t##length##millimeters,k##length##millimeters,x##length##millimeters,y##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| d##length##millimeters | Member depth | millimeters |
| b##length##millimeters, | Width of Flange | millimeters |
| t##length##millimeters, | Member thickness | millimeters |
| k##length##millimeters, | Distance from outer edge to end of radius | millimeters |
| x##length##millimeters, | Distance from edge to centroid in X-direction | millimeters |
| y##length##millimeters | Distance from edge to centroid in Y-direction | millimeters |

LL-Double Angle

Use M_L-Angle.txt as template

,Legs##other##,Space##length##millimeters,W##other##,A##area##millimeters,d##length##millimeters,b##length##millimeters,t##length##millimeters,y##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| Legs##other##, | A text identifier describing the orientation of the two angles. For example “LLBB” signifies Long Leg Back to Back | Text |
| Space##length##millimeters, | Distance (or gap) between the two angles | millimeters |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| d##length##millimeters | Member depth | millimeters |
| b##length##millimeters, | Width of Flange | millimeters |
| t##length##millimeters, | Member thickness | millimeters |
| y##length##millimeters | Distance from edge to centroid in Y-direction | millimeters |

Hollow Tube

Use M_HSS-Hollow Structural Section.txt as template

,Shape##other##,W##other##,A##area##millimeters,Ht##length##millimeters,b##length##millimeters,t##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| Shape##other##, | Text identifier indicating either “Rectangular” or “Square” | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| Ht##length##millimeters, | Height of member | millimeters |
| b##length##millimeters, | Width of Member | millimeters |
| t##length##millimeters | Thickness | millimeters |

Circular Hollow Tube

Use M_HSS-Round Structural Tubing.txt as template

,W##other##,A##area##millimeters,OD##length##millimeters,t##length##millimeters

| Fields | Descriptions | Units |
| --- | --- | --- |
| , | Type Name (or Section Name) | Text |
| W##other##, | Weight per length of member | Kn/m |
| A##area##millimeters, | Cross-Sectional Area | millimeters |
| OD##length##millimeters, | Outside Diameter | millimeters |
| t##length##millimeters | Thickness | millimeters |

Pipe Column

M_Pipe-Column.rfa

| Required Information |  |  |
| --- | --- | --- |
| Type | Depth (d) | Width (b) |
|  | mm | mm |

Open Web Steel Joist

M_K-Series Bar Joist-Angle Web.rfa, M_K-Series Bar Joist-Rod Web.rfa, M_DLH-Series Bar Joist.rfa, M_LH-Series Bar Joist.rfa

NOTE:  The Open Web Steel Joist is depicted in elevation and section below.

| Required Information |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Extended End | Seat Length | End Panel | Depth | Panel | Web Thickness | Cord Depth |
| text | mm | mm | mm | mm | mm | mm | mm |

# Wood Shapes

- Columns- Lumber

- M_Dimension Lumber-Column.rfa

| Required Information |  |  |
| --- | --- | --- |
| Type | Depth (d) | Width (b) |
| text | mm | mm |

- Columns- Timber

- M_Timber-Column.rfa

| Required Information |  |  |
| --- | --- | --- |
| Type | Depth (d) | Width (b) |
| text | mm | mm |

- Beam- Lumber

- M_Dimension Lumber.rfa

| Required Information |  |  |
| --- | --- | --- |
| Type | Depth (d) | Width (b) |
| text | mm | mm |

- Plywood Web Joist

- M_Plywood Web Joist.rfa

| Required Information |  |  |
| --- | --- | --- |
| Type | Depth (d) | Width (b) |
| text | mm | mm |

- Beam- Timber

- M_Timber.rfa

| Required Information |  |  |
| --- | --- | --- |
| Type | Depth (d) | Width (b) |
| text | mm | mm |

- Open Web Wood Joist

- M_Wood Open Web Joist.rfa

- NOTE:  The Wood Open Web Joist is given in elevation and section below.

| Required Information |  |  |  |
| --- | --- | --- | --- |
| Type | Depth | Chord Depth | Chord Width |
| text | mm | mm | mm |